化学品安全数据单

一、标识

物品名称：原甲酸三乙酯 /Triethyl orthoformate。 其它名称：/ 使用建议及使用限制：作为溶剂。在涂料中，作为专业涂料水分清除剂。 制造商名称：山东未名生物医药股份有限公司 地址： 淄博市张店区朝阳路18号 / 邮编： 255068 电子邮件地址：office@wanchang.com 企业电话号码： 0086-0533-2988888 企业传真号码： 0086-0533-2091578 企业应急电话： 0086-0533-2091588 国家应急电话： 0086-0532-3889090

二、危险标识

物质或混合物的分类： 易燃液体类别3。 全球统一制度标签要素，包括防范说明： 信号词：警告。 危险说明：易燃液体和蒸气。 防范说明： 预防：远离热源/火花/明火/热表面。禁止吸烟。保持容器密闭。容器和接收设备接地/等势联接。使用防爆的电气/通风/照明/设备。只能使用不产生火花的工具。采取防止静电放电的措施。戴防护手套/穿防护服/戴防护眼罩/戴防护面具。 应急：如误吞咽：漱口。不要诱导呕吐。如皮肤(或头发)沾染：立即脱掉所有沾染的衣服。用水清洗皮肤/淋浴。沾染的衣服清洗后方可重新使用。如误吸入：将人转移到空气新鲜处，保持呼吸舒适体位。立即呼叫解毒中心或医生。如进入眼睛：用水小心冲洗几分钟。如戴隐形眼镜并可方便地取出，取出隐形眼镜。继续冲洗。火灾时：使用合适的灭火器灭火。 贮存：存放在通风良好的地方。保持低温。 处置：按照相关规章处置内装物和容器。 不导致分类的其他危险（例如尘爆危险）或不为全球统一制度覆盖的其他危险：/

三、组成/成分信息

| 化学名称 | 化学文摘社登记号码（CAS No.） | 含量% |
| --- | --- | --- |
| 原甲酸三乙酯 | 122-51-0 | 99.20% |

四、急救措施

必要的急救措施 吸入：如果吸入,请将患者移到新鲜空气处。如果停止了呼吸,给予人工呼吸。求医。 皮肤接触：用肥皂和大量的水冲洗。求医。 眼睛接触：用大量水彻底冲洗至少15分钟。就医。 食入：禁止催吐。如果病人发生呕吐，使病人倾斜或左边侧卧(如果可能，保持低头体位)以保持呼吸道通畅，防止呛吸。小心看护病人。严禁对嗜睡或神志不清的病人给饮液体，如意识不清的病人。 最重要的急性和延迟症状/效应：中枢神经系统机能降低, 昏睡, 麻醉, 贫血。 必要时注明立即就医及所需的特殊治疗：/

五、消防措施

适用和不适用的灭火剂：抗醇泡沫。化学干粉。BCF(许可时)。二氧化碳。喷水或水雾-仅适于大火。 化学品产生的具体危险：液体和蒸气易燃。受热或接触明火，有中等程度的火灾危害。蒸气与空气能形成爆炸性混合物。受热或接触明火，有中等程度的爆炸性危害。蒸气能飘散相当长距离接触到点火源处。受热能引起膨胀或分解，导致容器急剧破裂。燃烧时能产生有毒的一氧化碳(CO)气体。 消防人员的特殊防护行为：避免物质与氧化剂接触。

六、意外释放措施

人身防范、保护设备和应急程序：使用个人防护设备。防止吸入蒸汽、气雾或气体。保证充分的通风。移去所有火源。将人员撤离到安全区域。防范蒸汽积累达到可爆炸的浓度,蒸汽能在低洼处积聚。 环境防范措施：在确保安全的条件下,采取措施防止进一步的泄漏或溢出。不要让产物进入下水道。 抑制和清理的方法和材料：用防电真空清洁器或湿的刷子将溢出物收集起来并放置到容器中去,根据当地规定处理(见第13部分)。

七、操作与储存

安全操作的防范措施：避免所有的个体接触，包括吸入。当有接触危险时，穿戴防护服。在通风良好的区域使用本物质。防止本品在坑凹处汇集。在未作空气检测之前，不得进入封闭空间内。禁止吸烟、外露灯光、受热或点火源。防止产生静电。禁止用塑料桶。 安全存储的条件，包括任何不相容性：储存于原装容器中，置于许可的易燃液体储存场所。禁止存放在凹坑、洼地、地下室或者气体能够汇聚的场所。禁止吸烟、外露灯光、受热或接触点火源。保持容器密封。远离禁忌物质，在凉爽、干燥、通风良好的场所储存。

八、接触控制/人身保护

控制参数：/ 工程控制：对易燃液体和易燃气体，可能需要局部通风系统或工艺围栏通风系统。通风设备应防爆。 个人防护设备 眼睛防护：带侧边的安全护目镜。化学护目镜。隐形眼镜可能会造成一种特殊危害；软的隐形眼镜可能会吸收和富集刺激物。 皮肤防护：戴化学防护手套(如聚氯乙烯 PVC)。穿安全鞋或安全靴(如橡胶材料)。防渗透的衣服, 阻燃防静电防护服,防护设备的类型必须根据特定工作场所中的危险物的浓度和含量来选择。 呼吸防护：呼吸器种类和型号的选择取决于呼吸区域污染物的等级以及污染物的化学性质。

九、物理及化学性质

| 外观（物理状态、颜色等） 气味 气味阙值 pH值 熔点/凝固点 初始沸点和沸腾范围 闪点 蒸发速率 易燃性（固态、气态） 上下易燃极限或爆炸极限 蒸气压力 蒸气密度 相对密度 可溶性 分配系数：n-辛醇/水 自动点火温度 分解温度 粘度 | 无色透明液体。 刺激性气味。 / / -18°C。 146°C。 30°C。 / / / / >1（空气=1）。 0.89（水=1）。 / / / / / |
| --- | --- |

十、稳定及反应性

反应性：/ 化学稳定性：/ 危险反应的可能性：/ 应避免的条件：热，火焰和火花。极端的温度和直接日光。 不相容的物质和材料：强氧化剂。 危险的分解产物：碳氧化物。

十一、毒理学信息

急性毒性效应： 吸入：吸入可能有害。可能引起呼吸道刺激。蒸气可引起睡意和眩昏。 食入：意外食入该物质可对个体健康造成伤害。吞咽液体可呛入肺内并有化学性肺炎的危险，可能导致严重的后果。 皮肤：通过皮肤吸收可能是有害的。可能引起皮肤刺激。 眼睛：本物质可能会刺激人的眼睛。 慢性毒性或长期毒性效应：长期接触呼吸道刺激物可能会导致气管疾病，包括呼吸困难和相关身体组织的疾病。 毒性的数值度量（如急性毒性估计值）：/

十二、生态信息

生态毒性:/ 持久性及降解性：水/土壤：高。 生物蓄积性：轻微。 在土壤中的流动性：高。 其它不利效应：/

十三、处置考虑

处置方法：尽可能回收本物质。如果不能确定有合适的处理或废弃处置设备，联系制造商有关回收方法，或联系当地或地区的废物管理部门有关废弃方法。按如下方法废弃处理：在有许可证的填埋处进行掩埋或在有许可证的焚化场进行焚化(与适当的可燃物质混合后)。对空的容器进行去污处理。遵守所有的标注规定，直至容器被清洗或销毁为止。

十四、运输信息

联合国编号：2524 联合国运输名称：原甲酸乙酯。 运输危险种类：3 包装类别：III。 环境危险：/ 使用者的特殊防范措施：/

十五、管理信息

国内化学品安全管理法规： 原甲酸三乙酯 （CAS：122-51-0）出现在以下法规中：中国现有化学物质名录。本安全数据单遵照了以下相关国家标准：GB16483-2008，GB13690-2009， GB6944-2005，GB/T15098-2008，GB18218-2009，GB15258-2009，GB6944-2005，GB190-2009，GB191-2009，GB12268-2008，GA57-1993，GB/T 15098-2008，GBZ 2-2007以及相关法规：《危险货物运输管理规则》、《危险化学品安全管理条例》、联合国《关于危险货物运输的建议书》（简称 UN RTDG）。

十六、其它信息

| 参考文献 | 联合国《关于危险货物运输的建议书.规章范本》 联合国《全球化学品统一分类和标签制度》 |
| --- | --- |
| 制表日期 | 2013年01月04日 |

注1：当产品为含有两种以上危险物质的混合物时，应依据其混合后的危险性，制作安全数据单。

注2：制造商/供应商应根据实际情况确保安全数据单所含信息的正确性，并适时更新。

注3：如由于产品特性而不存在或不可得某些信息时（如固体不存在沸点），应在表格中以“/”标识。

Chemical Safety Data Sheet

SECTION 1  IDENTIFICATION

Product Name: Triethyl orthoformate。 Other Name: / Recommended use of the chemical and restrictions on use: As a solvent in coatings. As a moisture scavenger in specialist coatings. Supplier’s details: SHANDONG SINOBIOWAY BIOMEDICINE CO.,LTD Address: 18 Chaoyang Road, Nanding Town, Zhangdian, Zibo, Shandong, China. Zip:255068 # Corporate Phone Number: 0086-533-2988888 Enterprise Fax Number: 0086-533-2091568 Emergency phone number: 0086-533-2091588 National Emergency Telephone: 0086-532-3889090

SECTION 2  HAZARDS IDENTIFICATION

Classification of the substance or mixture Flammable Liquid Category 3. GHS Label elements, including precautionary statements Signal word: Warning. Hazard statement(s): Flammable liquid and vapor. Precautionary statement(s): Prevention: Keep away from heat/sparks/open flames/hot surfaces. No smoking. Keep container tightly closed. Ground/Bond container and receiving equipment. Use explosion-proof electrical/ventilating/ lighting/equipment. Use only non-sparking tools. Take precautionary measures against static discharge. Wear protective gloves/protective clothing/eye protection/face protection. Response: IF SWALLOWED: Rinse mouth. Do NOT induce vomiting. IF ON SKIN (or hair): Take off immediately all contaminated clothing. Rinse skin with water/ shower. Wash contaminated clothing before reuse. IF INHALED: Remove person to fresh air and keep comfortable for breathing. Immediately call a POISON CENTER/doctor. Specific treatment (see safety sheet on this label). IF IN EYES: Rinse cautiously with water for several minutes. Remove contact lenses, if present and easy to do. Continue rinsing. In case of fire: Use Suitable fire extinguisher to extinguish. Storage：Store in well-ventilated place. Keep cool. Disposal: Dispose of contents/container to relevant regulations. Other hazards which do not result in classification: /

SECTION 3  COMPOSITION/INFORMATION ON INGREDIENTS

| Chemical Name | CAS No. | Concentration% |
| --- | --- | --- |
| Triethyl orthoformate | 122-51-0 | 99.0% |

SECTION 4  FIRST AID MEASURES

Description of necessary first aid measures If inhaled: If fumes or combustion products are inhaled remove from contaminated area. Lay patient down. Keep warm and rested. In case of skin contact: Immediately remove all contaminated clothing, including footwear. Flush skin and hair with running water (and soap if available). In case of eye contact: If this product comes in contact with the eyes: Wash out immediately with fresh running water. If swallowed: If swallowed do NOT induce vomiting. If vomiting occurs, lean patient forward or place on left side (head-down position, if possible) to maintain open airway and prevent aspiration. Observe the patient carefully. Most important symptoms and effects, both acute and delayed: / Indication of immediate medical attention and special treatment needed：/

SECTION 5  FIREFIGHTING MEASURES

Suitable extinguishing media: Alcohol stable foam. Dry chemical powder. BCF (where regulations permit). Carbon dioxide. Water spray or fog - Large fires only. Special hazards arising from the chemical: Liquid and vapour are flammable. Moderate fire hazard when exposed to heat or flame. Vapour forms an explosive mixture with air. Moderate explosion hazard when exposed to heat or flame. Vapour may travel a considerable distance to source of ignition. Special protective actions for fire-fighters: Avoid contamination with oxidizing agents .

SECTION 6  ACCIDENTAL RELEASE MEASURES

Personal precautions, protective equipment and emergency procedures: Remove all ignition sources. Clean up all spills immediately. Avoid breathing vapours and contact with skin and eyes. Control personal contact with the substance, by using protective equipment. Environmental precautions: Prevent further leakage or spillage if safe to do so. Do not let product enter drains. Methods and materials for containment and cleaning up: Contain spill with sand, earth or vermiculite. Use only spark-free shovels and explosion proof equipment. Collect recoverable product into labeled containers for recycling. Absorb remaining product with sand, earth or vermiculite.

SECTION 7  HANDLING AND STORAGE

Precautions for safe handling: Containers, even those that have been emptied, may contain explosive vapours. Do NOT cut, drill, grind, weld or perform similar operations on or near containers. DO NOT allow clothing wet with material to stay in contact with skin. Avoid all personal contact, including inhalation. Wear protective clothing when risk of overexposure occurs. Use in a well-ventilated area. Prevent concentration in hollows and sumps. DO NOT enter confined spaces until atmosphere has been checked. Avoid smoking, naked lights or ignition sources. Avoid generation of static electricity. Conditions for safe storage, including any incompatibilities: Store in original containers in approved flammable liquid storage area. Store away from incompatible materials in a cool, dry, well-ventilated area. DO NOT store in pits, depressions, basements or areas where vapours may be trapped. No smoking, naked lights, heat or ignition sources.

SECTION 8  EXPOSURE CONTROLS/PERSONAL PROTECTION

Control parameters: / Appropriate engineering controls: For flammable liquids and flammable gases, local exhaust ventilation or a process enclosure ventilation system may be required. Ventilation equipment should be explosion-resistant. Personal protective equipment Eye/face protection: Safety glasses with side shields. Chemical goggles. Contact lenses may pose a special hazard; soft contact lenses may absorb and concentrate irritants. Skin protection: Wear chemical protective gloves, eg. PVC. Wear safety footwear or safety gumboots, eg. Rubber. Impervious clothing, Flame retardant antistatic protective clothing, The type of protective equipment must be selected according to the concentration and amount of the dangerous substance at the specific workplace. Respiratory protection: Selection of the Class and Type of respirator will depend upon the level of breathing zone contaminant and the chemical nature of the contaminant.

SECTION 9  PHYSICAL AND CHEMICAL PROPERTIES

| Appearance Odour Odour Threshold pH Melting point/freezing point Initial boiling point and boiling range Flash point Evaporation rate Flammability (solid, gas) Upper/lower flammability or explosive limits Vapour pressure Vapour density Relative density Water solubility Partition coefficient: noctanol/water Autoignition temperature Decomposition temperature Viscosity | colorless clear liquid Pungent odor. / / -18°C 146°C 30°C / / / / / >1（air=1）。 0.89（water=1）。 / / / / |
| --- | --- |

SECTION 10  STABILITY AND REACTIVITY

Reactivity: / Chemical stability: / Possibility of hazardous reactions: / Conditions to avoid: Heat, flames and sparks. Extremes of temperature and direct sunlight. Presence of incompatible materials. Incompatible materials: Strong oxidizing agents. Hazardous decomposition products: carbon oxides.

SECTION 11  TOXICOLOGICAL INFORMATION

Acute health effects Inhalation: The material can cause respiratory irritation in some persons. The body's response to such irritation can cause further lung damage. Ingestion: Swallowing of the liquid may cause aspiration into the lungs with the risk of chemical pneumonitis; serious consequences may result. (ICSC13733). Accidental ingestion of the material may be damaging to the health of the individual. Skin: May be harmful if absorbed through skin. May cause skin irritation. Eyes: This material may produce eye irritation in some persons and produce eye damage 24 hours or more after instillation. Moderate inflammation may be expected with redness; conjunctivitis may occur with prolonged exposure. Chronic health effects: Long-term exposure to respiratory irritants may result in disease of the airways involving difficult breathing and related systemic problems. Substance accumulation, in the human body, may occur and may cause some concern following repeated or long-term occupational exposure. Numerical measures of toxicity(such as acute toxicity estimates): /

SECTION 12  ECOLOGICAL INFORMATION

Toxicity: / Persistence and degradability: Water/Soil: High. Bioaccumulative potential: LOW. Mobility in soil: HIGH. Other adverse effects: /

SECTION 13  DISPOSAL CONSIDERATIONS

Disposal methods: Recycle wherever possible. Consult manufacturer for recycling options or consult local or regional waste management authority for disposal if no suitable treatment or disposal facility can be identified. Dispose of by: burial in a land-fill specifically licensed to accept chemical and / or pharmaceutical wastes or Incineration in a licensed apparatus (after admixture with suitable combustible material). Decontaminate empty containers. Observe all label safeguards until containers are cleaned and destroyed.

SECTION 14  TRANSPORT INFORMATION

UN number: 2524. UN proper shipping name: ETHYL ORTHOFORMATE. Transport hazard class(es): 3. Packaging group: III Environmental hazards: / Special precautions for user: /

SECTION 15  REGULATORY INFORMATION

Regulations： Triethyl orthoformate (CAS: 122-51-0) is found on the following regulatory lists: "China Inventory of Existing Chemical Substances". This safety data sheet is in compliance with the following national standards: GB16483-2008，GB13690-2009， GB6944-2005，GB/T15098-2008，GB18218-2009，GB15258-2009，GB6944-2005，GB190-2009，GB191-2009，GB12268-2008，GA57-1993，GB/T 15098-2008，GBZ 2-2007as well as the following national regulations: Dangerous Goods Transport Administrative Regulation, Dangerous Chemicals Safety Administrative Regulation, United Nations Regulations on the Transport of Dangerous Goods (UN RTDG)

SECTION 16  OTHER INFORMATION

| References | “Model Regulations on the Transport of Dangerous Goods” “The Globally Harmonized System of Classification and Labeling of Chemicals” |
| --- | --- |
| Form Date | 04-Jan-2013 |

Note 1: When products contain two or more hazardous substances, Safety Data Sheets should be prepared based on the risk of the mixture.

Note 2: Manufacturer / supplier should ensure the correctness of the information contained in the safety data sheets, and updated in a timely manner.

Note 3: As a result of product features without the existence of certain information or no data available (such as boiling point does not exist for the solid) in the table with "/" logo.