Material Safety Data Sheet

DEAC

1. IDENTIFICATION OF THE SUBSTANCE/PREPARATION AND SUPPLIER

Product Identifier Diethylaluminum chloride, 98 %

Supplier Jiangsu MO Opto-Electronic Material CO., Ltd. No. 7, Tonggang Road, Zhenjiang New District, Dagang，Zhenjiang, 212000, P. R. China. Tel.: (86)-511-88053387 Fax: (86)-511-85575111 http://www.js-mo.com/china/index.asp

Emergency telephone +86 511 88053387

Intended use co-catalyst for olefin polymerization; Catalyst for chemical reactions.

Date of last issue / Revision number 2014/08/24/1.01

Chemical family Metal alkyl

2. HAZARDS IDENTIFICATION

Designation EXTREMELY FLAMMABLE ~ TOXIC ~ CORROSIVE Risk Phrases R12 Extremely flammable. R14 Reacts violently with water. R17 Spontaneously flammable in air. R23/24/25 Toxic by inhalation, in contact with skin and if swallowed. R34 Causes burns.

3. COMPOSITION/INFORMATION ON INGREDIENTS

| Information on hazardous ingredients |  |  |  |
| --- | --- | --- | --- |
| Chemical description Diethylaluminum chloride, 98 % |  |  |  |
| Composition / information on ingredients |  |  |  |
| Number | % w/w | CAS-number | Chemical name |
| 1 | approx. 99 | 96-10-6 | Diethylaluminum chloride |

4. FIRST AID MEASURES

Symptoms and effects Causes severe burns. Causes injury to the cornea and eyelids. Risk of serious damage to eyes.

First aid

General Call a physician immediately.

Inhalation Remove to fresh air. If not breathing, give artificial respiration. Oxygen may additionally be given, by trained personnel, if it is available. Get medical attention immediately.

Skin While wearing impervious gloves and air-tight safety goggles, immediately start continuous flushing of all affected areas on the victim with water for at least 15 minutes. If victim is wearing air-tight safety goggles, do not remove them. Remove contaminated clothing and shoes. If clothing is stuck to the skin after flushing with water, do not remove it. Get medical attention immediately. Wash or destroy clothing. Thoroughly clean or destroy contaminated shoes.

Eye Immediately start continuous flushing of eyes with water for at least 15 minutes. If easy to do, contact lenses should be removed during the flushing, by trained personnel. Hold the eyelids apart during the flushing to ensure rinsing the entire surface of the eye and lids with water. Get medical attention immediately.

Ingestion DO NOT induce vomiting. Get medical attention immediately by calling a physician or a poison control center. If victim is conscious and alert, give a cupful of water. Never give anything by mouth to an unconscious or convulsing person. If vomiting occurs, the patient should lie on their left side while vomiting to reduce the risk of aspiration.

Advice to physician Persons with pre-existing skin, eye, or respiratory disease may be at increased risk from the irritant or allergic properties of this material. Irrigate burn area with large amounts of water to decontaminate, if not already done. Chemical burns on the skin should then be treated like thermal burns. Skin reactions may take 24-48 hours to develop. If eyes are affected, flush eyes with buffered or plain irrigating solutions for at least 15 minutes, if not already done. If any ulceration or conjunctival injury is present, have an ophthalmologist examine the patient. Application of cool water helps relieve pain and swelling of both the skin and eyes. If swallowed, do not induce vomiting. Give patient plenty of water to drink. Ingestion of this corrosive material may cause severe ulceration, inflammation, and possible perforation of the gastrointestinal tract. Maintain adequate airway. Aspiration during induced emesis can result in severe lung injury. Contact a Poison Control Center for additional treatment information. Treat any additional effects symptomatically.

5. FIRE-FIGHTING MEASURES

Extinguishing media vermiculite, dry chemical powder, dry sand.

Unsuitable extinguishing media Never use water !!! See also Section: Other information. halones

Hazardous decomposition / combustion products Products of complete combustion are carbon dioxide, water, aluminum oxide and hydrogen chloride. Additionally, products of incomplete combustion may include carbon monoxide, elemental carbon and hydrocarbons (alkanes and alkenes).

Protective equipment Wear a standard aluminised firefighting suit. Wear self contained breathing apparatus.

Other information Waterspray may only be used by experienced fire fighters. Cool closed containers with water.

Fire and explosion hazard CAUTION: reignition may occur. In case of fire and/or explosion do not breathe fumes.

6. ACCIDENTAL RELEASE MEASURES

Personal precautions Do not breathe fumes/vapor. Avoid contact with skin and eyes. For personal protection see Section 8.

Environmental precautions Do not allow to enter drains or water courses.

Methods for cleaning up Isolate spill area. After fire has been extinguished or has been allowed to burn out completely, wait CONSIDERABLE TIME (until smoke is no longer observed). After that, carefully wash spill area with a waterspray.

Other information Ignition will occur.

7. HANDLING AND STORAGE

Handling When using do not eat, drink or smoke. Handle in well ventilated areas. Take precautionary measures against static discharges. Apply grounding when transferring from one container to another. Avoid contact with moisture and water. Keep under nitrogen. Handle only in closed system. During sampling, disconnecting lines or opening connections, an aluminised suit should be worn. Avoid contact with skin and eyes.

Fire and explosion prevention Spontaneously flammable in air

Storage requirements Store in accordance with local/national regulations. Keep away from food, drink and animal feedingstuffs. Keep under dry nitrogen containing less than 10 ppm oxygen. Protect product from moisture and wet air.Keep container tightly closed and in a well-ventilated place.

Other information Wash hands thoroughly after handling or contact. Keep working clothing separately and do not take them home.

8. EXPOSURE CONTROLS/PERSONAL PROTECTION

Engineering controls Ensure good ventilation and local exhaustion of the working area.

Personal protection

Respiratory In case of insufficient ventilation wear suitable respiratory equipment (respirator with Filter B / E / P2).

Hand impervious gloves.

Eye Safety glasses and a full face shield.

Skin and body Aluminised suit and protective boots (For further advice contact manufacturer).

Other information Launder clothes before reuse.

In this country no exposure limit has been established

9. PHYSICAL AND CHEMICAL PROPERTIES

Appearance liquid

Color clear, colorless

Odor not applicable

Boiling point/range 260.6 °F (127 °C) (6.67 kPa )

Melting point/range -121 °F (-85 °C)

Flash point not applicable

Flammability pyrophoric ( paper char test ) Reacts explosively with water

Explosive properties not applicable

Oxidizing properties not applicable

Vapor pressure 0.4 kPa (60 °C )

Density 961 kg/m³ (25 °C )

Bulk density not relevant

Solubility in water Reacts explosively with water

Solubility in other solvents Miscible with saturated aliphatic and aromatic hydrocarbons

pH value acidic character

Partition coefficient n-octanol/water not relevant

Relative vapor density (air=1) not determined

Viscosity 1.4 mPa.s (30 °C )

Autoignition temperature Spontaneously flammable in air

Explosion limits not applicable

10. STABILITY AND REACTIVITY

Conditions to avoid

To maintain quality store in original closed container below: 180 °C.

exothermic).

Stability Stable under recommended storage and handling conditions (see section 7).

Incompatibles Avoid contact with moisture and water, alcohols, acids, organic halides and oxygen containing compounds.

Polymerization Polymerization does not occur.

Hazardous decomposition products Hydrochloric acid, Aluminum, hydrocarbons

11. TOXICOLOGICAL INFORMATION

Diethylaluminum chloride

Acute toxicity

Oral LD50 No data available

Irritation

Skin Corrosive

Eye Corrosive; Risk of serious damage to eyes

Respiratory Corrosive

12. ECOLOGICAL INFORMATION

No data available

13. DISPOSAL CONSIDERATIONS

Product Refer to manufacturer/supplier for information on recovery/recycling. Waste disposal in accordance with regulations (most probably controlled incineration).

Contaminated packaging According to local regulations.

Other information For further advice contact manufacturer.

14. TRANSPORT INFORMATION

Land transport

Class 4.2

Packing group I

UN number 3394

Proper Shipping Name ORGANOMETALLIC SUBSTANCE, LIQUID, PYROPHORIC,WATER-REACTIVE ( Diethylaluminum chloride )

Required labels 4.2 + 4.3

Subsidiary risk 4.3

Sea transport (IMO / IMDG-code)

Class 4.2

Packing group I

UN number 3394

EMS F-G, S-M

Marine pollutant no

Proper Shipping Name ORGANOMETALLIC SUBSTANCE, LIQUID, PYROPHORIC, WATER-REACTIVE ( Diethylaluminum chloride )

Other information Label(s) : 4.2 + 4.3

Air transport (ICAO-TI / IATA-DGR)

UN number Forbidden

15. REGULATORY INFORMATION

Product label name Diethylaluminum chloride, 99%

| Hazard classes |  |
| --- | --- |
| Description | Applicable |
| Chinese classification | This product is classified CORROSIVE in accordance with "Chinese regulation of classification and labeling of dangerous chemicals"(GB 13690-92). |
| Chinese classification | This product is classified SELF-IGNITION MATERIAL in accordance with "Chinese regulation of classification and labeling of dangerous chemicals"(GB 13690-92). |
| Dangerous Chemical Regulation | Regulation on Safe Management of Dangerous Chemicals (take effect from March 16, 2002) gives out the corresponding requirements on dangerous chemicals in the areas of safe production, use, storage, transportation, load and unload etc.. |

| R(isk) phrase(s) (EU classification) |  |
| --- | --- |
| Code | Description |
| R14. | Reacts violently with water. |
| R17. | Spontaneously flammable in air. |
| R35. | Causes severe burns. |

| S(afety) phrase(s) (EU classification) |  |
| --- | --- |
| Code | Description |
| S06B. | Keep under nitrogen. |
| S16. | Keep away from sources of ignition - No smoking. |
| S24/25. | Avoid contact with skin and eyes. |
| S36/37/39. | Wear suitable protective clothing, gloves and eye/face protection. |
| S43B. | In case of fire, use dry chemical powder; never use water. |
| S45. | In case of accident or if you feel unwell, seek medical advice immediately (show the label where possible). |

| Symbol(s) (EU classification) |  |
| --- | --- |
| CORROSIVE | HIGHLY FLAMMABLE |

Other information Substance and/or product listed in Directive 96/82/EC.

16. OTHER INFORMATION

| R-phrase information |  |  |
| --- | --- | --- |
| Chemical name | R(isk) phrase(s) (EU classification) |  |
| Diethylaluminum chloride | R14 R17 R35 | Reacts violently with water Spontaneously flammable in air Causes severe burns |

History

Date of printing/ pdf file generated 2014/08/24

Revision 1.01

Composed by Jiangsu MO Opto-electronic Material Co. Lted. (P. R. China). T +86-511-88050387.

Changes were made in section 2, GHS classification (EU)

This information only concerns the above mentioned product and does not need to be valid if used with other product(s) or in any process. The information is to our best present knowledge correct and complete and is given in good faith but without warranty. It remains the user's own responsibility to make sure that the information is appropriate and complete for his special use of this product.