CERTIFICATE OF ANALYSIS

| Product Name | Carvedilol | Batch Size | 100kg |
| --- | --- | --- | --- |
| Batch No. | KW0150502 | Report Date | 2015-05-02 |
| Manufacture Date | ##### 2015-05-02 | Expiration Date | 2018-05-01 |
| Test Item Specification Result Appearance White or almost white crystalline powder Qualified Solubility Practically insoluble in water, Slightly soluble in alcohol, Qualified Practically insoluble in dilute acids Identification -IR -Similar to standard Qualified -HPLC -Similar to standard Qualified Melting Point 113℃-116℃ 114.1℃-115.3℃ Heavy Metals ≤0.001% ＜0.001% Sulphated Ash ≤0.1% 0.02% Loss on drying ≤0.5% 0.36% ### Related substances Impurity A ≤0.2% 0.17% Impurity C ≤0.02% 0.01% Any other impurities ≤0.10% 0.07% Total impurities ≤0.50% 0.32% Residual solvent Ethanol ≤5000ppm 1980ppm Assay 99.0%-101.0% of C24H26N2O4, 99.57% CALCULATED ON DRIED BASIS CONCLUSION: CONFORM WITH SPECIFICATION AS PER EP5.1. |  |  |  |

Analyst:                   Checker:                   Supervisor: